MASTER RESEARCH GUIDE
Wearable / Smartphone Detection of Glucose and Insulin Resistance

# 1. Executive Summary

Goal: learn enough biology, wearable sensing, signal processing, machine learning and clinical validation to understand and reproduce published work, then formulate an original research question.

Key distinction: direct glucose sensing is different from non-invasive glucose estimation. A smartwatch may provide physiological signals from which a model predicts glucose or insulin resistance; that is not the same as directly measuring blood glucose.

Practical pathway: metabolic physiology → CGM → PPG/ECG/HRV/activity/sleep → signal processing → classical ML → deep learning → reproduction → original study.

# 2. The Main Literature

Metwally et al. — Insulin resistance prediction from wearables and routine blood biomarkers (Nature, 2026)

WEAR-ME study; n=1,165; wearable time-series + routine biomarkers; target HOMA-IR. Reported multimodal AUROC 0.80, sensitivity 76%, specificity 84%; independent validation cohort n=72.

LINK: https://www.nature.com/articles/s41586-026-10179-2

Jovanova et al. — Digital biomarkers for insulin resistance screening in daily life (medRxiv, 2026)

CGM glucose dynamics + smartwatch heart rate/HRV; Study 1 n=97 and external validation Study 2 n=61. Preprint, so treat results as preliminary.

LINK: https://www.medrxiv.org/content/10.64898/2026.05.20.26353669v1.full

Bent et al. — Engineering digital biomarkers of interstitial glucose from noninvasive smartwatches (npj Digital Medicine, 2021)

Directly relevant to smartwatch-derived digital biomarkers and glucose estimation.

LINK: https://www.nature.com/articles/s41746-021-00465-w

Dave et al. — Detection of Hypoglycemia and Hyperglycemia Using Noninvasive Wearable Sensors: ECG and Accelerometry (2022)

ECG + accelerometer; five participants in the analyzed dataset; CGM reference. Useful example of wearable signals plus ML.

LINK: https://pmc.ncbi.nlm.nih.gov/articles/PMC10973850/

Wang et al. — Design of a Wearable Finger PPG-Based Blood Glucose Monitor (2025)

Finger PPG using 660/880-nm LEDs with an MLP model. Useful for PPG hardware + ML.

LINK: https://pubmed.ncbi.nlm.nih.gov/40685463/

Jiang et al. — PPG-based glucose sensors: a review (2025)

Focused review of PPG-based glucose sensing, ML approaches and wearable implementation.

LINK: https://link.springer.com/article/10.1007/s10462-025-11379-4

Mansour et al. — Wearable devices for glucose monitoring: review of state-of-the-art technologies and emerging trends (2024)

Broad review of invasive, minimally invasive and non-invasive glucose monitoring, electronics and AI.

LINK: https://doi.org/10.1016/j.aej.2024.01.021

Huang et al. — Technological Advances of Wearable Device for Continuous Monitoring of In Vivo Glucose (ACS Sensors, 2024)

Technical review of wearable CGM and glucose-monitoring technologies.

LINK: https://pubs.acs.org/doi/10.1021/acssensors.3c01947

Min et al. — Minimally and non-invasive glucose monitoring: the road toward commercialization (2025)

Critical review of optical, millimeter-wave, ultrasound, bioimpedance, electrochemical and wearable approaches.

LINK: https://pubs.rsc.org/en/content/articlehtml/2025/sd/d4sd00360h

Non-Invasive Technologies in Wearable Glucose Monitoring: A Structured Overview for the Future (2026 review)

Recent overview of non-invasive wearable glucose methods and limitations including motion artifacts and clinical validation.

LINK: https://pmc.ncbi.nlm.nih.gov/articles/PMC13511495/

Non-invasive wearable electrochemical sensors for continuous glucose monitoring (2025 mini-review)

Review of epidermal electrochemical glucose sensors and alternative biofluids.

LINK: https://www.sciencedirect.com/science/article/pii/S1388248125000335

# 3. Recommended Reading Order

Read the 2024 wearable-glucose review first for the landscape.

Read the 2025 commercialization review to understand why non-invasive glucose is difficult.

Learn PPG, ECG, HR and HRV fundamentals.

Read Bent et al. (2021) on smartwatch digital biomarkers.

Read/reproduce Dave et al. (2022) on ECG + accelerometer + CGM.

Read Wang et al. (2025) and the 2025 PPG review.

Read Metwally et al. (2026) carefully, including methods and supplementary material.

Read Jovanova et al. (2026) and compare its CGM + smartwatch design.

Then move to newer deep-learning and wearable-foundation-model papers.

# 4. 60-Day Roadmap

| Days | Theme | Learn | Deliverable |
| --- | --- | --- | --- |
| Days 1–7 | Biology | Glucose, insulin, IR, HOMA-IR, OGTT, HbA1c, clamp | Explain the target and reference standard |
| Days 8–14 | Wearable signals | PPG, ECG, HRV, accelerometer, temperature, SpO2, sleep | Map signals to sensors |
| Days 15–21 | CGM | Interstitial vs blood glucose, electrochemical sensing, lag, calibration, noise | Build a CGM data pipeline |
| Days 22–30 | Signal processing | Filtering, artifact removal, segmentation, alignment, features | Create model-ready windows |
| Days 31–45 | ML/DL | Regression, RF, XGBoost, SVM, CNN, LSTM, Transformers | Build baseline models |
| Days 46–60 | Reproduction | Validation, leakage, metrics, limitations, research gaps | Reproduce 1–3 studies |

# 5. Biology Checklist

Glucose physiology

Insulin secretion/action

Insulin resistance vs sensitivity

Fasting glucose and insulin

HOMA-IR and limitations

HbA1c

OGTT

Hyperinsulinemic-euglycemic clamp

Postprandial glucose

Glucose variability

# 6. Wearable Signal Checklist

PPG: optical waveform, pulse morphology, pulse timing, motion artifacts, contact pressure.

ECG: R peaks, RR intervals, heart rate and HRV.

HRV: time/frequency-domain measures and confounding by sleep, exercise, stress, respiration and medications.

Accelerometer/gyroscope: movement, exercise and sedentary behavior.

Skin temperature: useful context but strongly affected by environment and contact.

SpO2: physiological context, not a glucose measurement.

Sleep: duration, timing, regularity and stages.

# 7. CGM and Glucose-Sensing Checklist

Blood vs interstitial glucose

Electrochemical CGM

Sampling interval

Interstitial lag

Calibration/drift

Noise and missing data

Meal excursions

Rate of change

Time above/below range

Mean/SD/coefficient of variation

Post-meal peak and recovery

Optical/Raman/NIR/bioimpedance/sweat/tear/saliva approaches

# 8. Signal Processing and ML

Sampling/resampling

Low-pass/band-pass filtering

Peak detection

Signal-quality assessment

Motion-artifact handling

Windowing/segmentation

Normalization

Missing-data handling

Timestamp alignment

Feature extraction

Participant-level aggregation

Leakage prevention

Linear/logistic regression

Random Forest

XGBoost

SVM

1D CNN

LSTM/GRU

Transformers

Multimodal fusion

Personalized vs subject-independent models

# 9. Recommended Research Architecture

Smartwatch: PPG + HR + HRV + accelerometer + sleep + temperature where available.

Phone: meal/exercise/context data and synchronization.

CGM: reference glucose time series during model development.

Pipeline: timestamp alignment → quality control → feature extraction → ML/DL → glucose-dynamics or IR prediction → validation.

A sensible progression is CGM + smartwatch first; once validated, investigate whether CGM can be removed from the inference stage.

# 10. Paper Reading Checklist

Research question

Population/sample size

Sensors/data

Ground truth

Synchronization

Preprocessing

Features

Model

Data split

Participant-independent validation

External validation

Metrics

Confidence intervals

Data-leakage risk

Limitations

Experiment that could address a limitation

# 11. Paper Extraction Spreadsheet Columns

Paper | Year | N | Population | Sensor | Signal | Ground truth | Features | Model | Validation | Main result | Research gap

# 12. Metrics to Learn

MAE

RMSE

MARD

AUROC

Sensitivity/specificity

Precision/recall

Calibration

Clarke Error Grid

Parkes Error Grid

Bland–Altman analysis

Confidence intervals

Subject-independent validation

External validation

# 13. Common Research Mistakes

Calling wearable prediction direct glucose measurement.

Randomly splitting windows from the same participant into train/test and claiming generalization.

Ignoring repeated measurements.

Using a very large deep model on a small dataset.

Reporting accuracy without population and validation context.

Comparing papers with different endpoints as if they were equivalent.

Ignoring PPG motion, skin contact, temperature and device differences.

Treating CGM as identical to simultaneous blood glucose.

Skipping a simple baseline.

Failing to document preprocessing.

# 14. Potential Research Questions

Can longitudinal PPG-derived features predict HOMA-IR?

Can multimodal smartwatch signals improve IR prediction over PPG alone?

Can CGM glucose dynamics + HRV/activity provide digital biomarkers of IR?

How well does a model trained on one wearable generalize to another?

How robust are models in real-world conditions?

Can subject-independent models approach personalized performance?

Which wearable signals add information after accounting for age, sex and BMI?

Can estimated glucose dynamics from wearable signals support IR screening without continuous CGM?

# 15. Python / Software Stack

Python

Jupyter

NumPy

pandas

SciPy

matplotlib

scikit-learn

XGBoost

PyTorch

Git/GitHub

Spreadsheet/database for literature extraction

# 16. Daily Study Routine

45–60 min reading

30–60 min concept study

45–90 min coding

10–15 min research notes

Weekly paper-spreadsheet update and one-page synthesis

# 17. 90-Day Extension

Days 1–60: core learning and reproduction.

Days 61–75: focused literature search around one research gap.

Days 76–90: define dataset, endpoint, inclusion criteria, preprocessing, model, validation strategy and study protocol.

For human-subject work, obtain appropriate institutional/ethics approval and define privacy/data-security procedures before data collection.

# 18. Success Criterion

You do not need to read every paper. A strong milestone is being able to take a published wearable-glucose/IR study, explain its biology, sensors, preprocessing, features, model, validation and limitations, reproduce a baseline, and formulate a testable research question.

# 19. Master Source List

Metwally et al. — Insulin resistance prediction from wearables and routine blood biomarkers (Nature, 2026)

https://www.nature.com/articles/s41586-026-10179-2

Jovanova et al. — Digital biomarkers for insulin resistance screening in daily life (medRxiv, 2026)

https://www.medrxiv.org/content/10.64898/2026.05.20.26353669v1.full

Bent et al. — Engineering digital biomarkers of interstitial glucose from noninvasive smartwatches (npj Digital Medicine, 2021)

https://www.nature.com/articles/s41746-021-00465-w

Dave et al. — Detection of Hypoglycemia and Hyperglycemia Using Noninvasive Wearable Sensors: ECG and Accelerometry (2022)

https://pmc.ncbi.nlm.nih.gov/articles/PMC10973850/

Wang et al. — Design of a Wearable Finger PPG-Based Blood Glucose Monitor (2025)

https://pubmed.ncbi.nlm.nih.gov/40685463/

Jiang et al. — PPG-based glucose sensors: a review (2025)

https://link.springer.com/article/10.1007/s10462-025-11379-4

Mansour et al. — Wearable devices for glucose monitoring: review of state-of-the-art technologies and emerging trends (2024)

https://doi.org/10.1016/j.aej.2024.01.021

Huang et al. — Technological Advances of Wearable Device for Continuous Monitoring of In Vivo Glucose (ACS Sensors, 2024)

https://pubs.acs.org/doi/10.1021/acssensors.3c01947

Min et al. — Minimally and non-invasive glucose monitoring: the road toward commercialization (2025)

https://pubs.rsc.org/en/content/articlehtml/2025/sd/d4sd00360h

Non-Invasive Technologies in Wearable Glucose Monitoring: A Structured Overview for the Future (2026 review)

https://pmc.ncbi.nlm.nih.gov/articles/PMC13511495/

Non-invasive wearable electrochemical sensors for continuous glucose monitoring (2025 mini-review)

https://www.sciencedirect.com/science/article/pii/S1388248125000335